This simple circuit uses an incandescent lamp to detect airflow. With the filament exposed to air, a constant current source is used to slightly heat the filament. As it is heated, the resistance increases. As air flows over the filament it cools down, thus lowering it's resistance. A comparator is used to detect this difference and light an LED. With a few changes, the circuit can be connected to a meter or ADC to provide an estimation on the amount of air flow.

### Schematic

### Parts

| Part; Total Qty.; Description; Substitutions. R1; 1; 100 Ohm 1/4W Resistor. R2; 1; 470 Ohm 1/4W Resistor. R3; 1; 10k 1/4W Resistor. R4; 1; 100K 1/4W Resistor. R5; 1; 1K 1/4W Resistor. C1; 1; 47uF Electrolytic Capacitor. U1; 1; 78L05 Voltage Regulator. U2; 1; LM339 Op Amp. L1; 1; #47 Incandescent lamp with glass removed (See "Notes"). D1; 1; LED. MISC; 1; Board, Wire, Sockets for ICs, etc.. |  |
| --- | --- |

### Notes

- The glass will have to be removed from L1 without breaking the filament. Wrap the glass in masking tape and it in a vise. Slowly crank down until the glass breaks, then remove the bulb and carefully peel back the tape. If the filament has broken, you will need another lamp.